Study on Topological structure of sandstone 3D pore networks

Guannan Liu 1,2 *,  Xiaopeng Pei 3 ,  Jishan Liu 1,2 ,  Dayu Ye 3 ,  Feng Gao 1,2 ,  Jianguo Wang 1,2

1Mechanics and Civil engineering Institute, China University of Mining and Technology, Xuzhou city, Jiangsu province 221000, China

2 State Key Laboratory for Geomechanics and Deep Underground Engineering, China University of Mining and Technology, Xuzhou city, Jiangsu province 221000, China

3 Chemical engineering Institute, China University of Mining and Technology, Xuzhou city, Jiangsu province 221000, China

* correspondence to Guannan Liu (guannanliu@cumt.edu.cn)

Abstract:  There are a large number of pores and throats in rock, the size and shape of which are different, and the connection status is complicated. Based on complex network theory and X ray CT scanning techniques, we take sandstone as an example to investigate the topological characteristics of sandstone pore networks. The results show that the distribution of the degree of the pores in sandstone meets single-peaked curve. The average path length increases with the increase of network size (number of nodes). And the nodes are ranked by the importance in the network. By analyzing the pore network modularity, it is concluded that the sandstone pore network has a homogeneous community structure, which means there is no dominant communities in the network. The paper try to provide a new perspective for the study of the mechanism of fluid storage and transport in rock and other porous materials.

Key words：Complex network theory   sandstone   network topology   Seepage   distribution of degree

- Introduction:

The pore morphology in rocks are different, cross-scale distributed, and the connection structure is complex[1-3].It has great theoretical and practical significance to study the characteristics of rock for the accurate and quantitative evaluation of rock permeability, revealing the mechanism of oil and gas accumulation and migration, and enhancing oil recovery[4-6].

In recent years, a lot of research on the statistical characteristics and evolution model of rock pore structure have been done by many scholars. Blunt and Raeini developed a generalized network extraction method for three-dimensional digital core to reduce the uncertainties of modeling[7] . Using a focused ion beam scanning electron microscope (FIB-SEM), Shaina K. [8] obtained a microscale shale digital core and compared it with conventional scanning electron microscopy in terms of porosity, organic content, and pore connectivity. Using statistical method, Hajizadeh took the CT scan image data of sandstone as a sample, and coupled the continuous two-dimensional multi-point statistical simulation to the multi-scale data extraction program, and proposed a random porous media reconstruction technique[9]. As in the traditional Darcy law is no longer suitable for dense rock, Civan[10] used the modified Darcy law to describe the gas migration in dense shale, in which the apparent permeability is a function of intrinsic permeability and porosity. Considering the basic concept of pore network model, a modeling method based on pore volume and throat searching is proposed by Zhang[11] , and combined with percolation theory, the permeability of different models is calculated. Yang J. [12] used fractal control function to describe the complex morphology of rock pore structure, and proposed a fractal reconstruction model of rock pore structure with improved simulated annealing algorithm.

At present, the characterization of pore connectivity structure is mainly studied from macro average perspective or by Eular number. The comprehensive topological characteristics of the porous network such as the degree of clustering and the choice of percolation path are still need to be studied. In this paper, from the point of view of complex network theory, aiming at the real sandstone pore network, the topological characteristics and correlation properties of pore throat network are analyzed to provide the basis for revealing the microscopic mechanism of porous rock seepage.

- Basic information of sandstone pore network

The data of sandstone core constructed by Professor Blunt of Imperial College London is used in our work. Image analysis uses Micro-CT produced by German Phoenix company was used to analysis the image, which equipped with a 1 m focus system with a view of 512 * 512 pixels with a 8 inch 16 bit detector. Four sandstone samples with porosities of 4.1%, 16.9%, 17.1% and 24.6% are used in our work. The three-dimensional sandstone pore network of the four digital cores are shown in Fig. 1[17-18] .

a. 14.1%                                 b. 16.9%

c. 17.1%                                 d. 24.6%

Fig. 1 Sandstone three-dimensional pore network (Bounding Box volume: 300μm by 300μm by 300μm. The pore network is extracted by the skeleton algorithm in the digital rock reconstructed from the X ray CT scanner)

Because the isolated pores do not influence the seepage process, they were removed during the pore network construction of sandstone. Based on the complex network theory, the basic information of the pore network structure of sandstone was analyzed, as shown in table 1. As shown in Table 1, the pore network properties of sandstone are mainly related to network size (nodal number) rather than sandstone porosity, When the scale of the network increases, the diameter of the network and the average path length show a general trend of growth.

Table 1 Basic information of pore network

| Porosity | Node Number |  |  | Edge Number |  | Average Degree |  | Network Diameter |  | Average Path length |  | Network Density |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14.1% |  | 1717 | 2824 |  | 3.289 |  | 25 |  | 10.151 |  | 0.002 |  |
| 16.9% |  | 8301 | 14381 |  | 3.465 |  | 46 |  | 16.469 |  | 0 |  |
| 17.1% |  | 8542 | 12432 |  | 2.962 |  | 50 |  | 19.428 |  | 0 |  |
| 24.6% |  | 1945 | 4697 |  | 4.83 |  | 22 |  | 8.466 |  | 0.002 |  |

- Sandstone pore networks structure

First, the degree distributions of nodes in the network are analyzed, as shown in figure 2. The results show that the degree distributions of the sandstone pore network are single-peaked curves. The sandstone pore networks with different porosity have characteristic values. When the porosities are 14.1%, 16.9% and 17.1%, the nodes which degree are 2 have a moderate proportion, when the porosity is 24.6%, the highest proportion of nodes are the nodes whose degree are 4. And when the degree exceeds these values, the number of nodes decreases abruptly.

Fig. 2 Degree distribution of sandstone three-dimensional pore network

In order to analyze the importance of individual nodes in sandstone pore networks, the eigenvector centrality is used to compute the network nodes. Make  as the index value of node , and  as the adjacency matrix of the network. For node , the centrality index is proportional to the exponential sum of all nodes connected to it。That is:

Where  is the set of nodes that are connected to node ,  is constant.

The result of the centrality of the eigenvector is shown in Fig.3. The abscissa in the figure is the node Id. As shown in Fig. 3, when the porosity of sandstone is between 14.1% and 24.6%, the eigenvector centrality of most nodes is less than 0.4. Only a small number of nodes have great centrality. In the process of oil and gas extraction, if the rock debris blocked these few pores, or these pore are closed by crustal stress, it may have an important impact on the entire percolation network, leading to a significant decline in resource recovery.

a. 14.1%                                                      b. 16.9%

c. 17.1%                                                      d. 24.6%

Fig. 3 Eigenvector centrality distribution of sandstone network node

To facilitate the intuitive understanding, the Fruchterman Reingold algorithm is used to distribute the pore nodes(Fig.1) uniformly in the circular surface. And the node centrality is sorted as shown in Fig. 4. Where the node size represents the node centrality, and the number represents the node Id.

a. 14.1%                                        b. 16.9%

c. 17.1%                                          d. 24.6%

Fig. 4 centrality sorting of sandstone pore nodes

The clustering coefficient is the coefficient that indicates the degree of node aggregation in a graph. In the network of reality, the higher the clustering coefficient, the more tend the nodes tend to be more closely connected. In the network, if the node  is connected to the node  and the node  is connected to the node , then the node  may be connected to the . This phenomenon reflects the connection density between nodes. In an undirected network, the clustering coefficients can be expressed as:

Where  Represents the number of adjacent nodes of the node ;  represents the number of edges that are connected among all adjacent nodes of the node . The clustering coefficient distribution of sandstone pore network is shown in Fig. 5.

a. 14.1%                                                     b. 16.9%

c. 17.1%                                                      d. 24.6%

Fig. 5 clustering coefficient distribution of sandstone three-dimensional pore network

Fig.5 shows that the clustering coefficients of different sandstone pore networks are mainly distributed between 0 and 0.5. The nodes with the clustering coefficient of 0 have the largest proportion, and the number of nodes decreases as the clustering coefficient increases. It shows that sandstone pore network has less connectivity than the global coupling network which clustering coefficient is 1. According to Fig. 2, the main reason is that the main pore connected to the sandstone pore is the peripheral pore node. In general, as the distance increases, the probability of connection between two nodes become weaker.

The degree of aggregation of the entire network can be represented by the average value of the clustering coefficients of all nodes, as shown in Fig. 6. Networks with higher average clustering coefficients have smaller average distances in different nodes. Fig. 6 shows that the average clustering coefficient is the largest when the porosity is 24.60% in the sampling sandstone, and as shown in Table 1, the average path length is minimum, that is, the network has the best connectivity among the four sandstone sample.

Fig. 6 Average clustering coefficient of sandstone pore network

The modular algorithm proposed by Blondel is used to analyze sandstone pore networks for community properties. We calculate the density of each sub network after partition, and then calculate the density of the sub network under the condition of random connected. There is a difference between these two densities, which represents the degree in which the sub network deviates from a random situation. The greater the value, the more dense the network is, compared with the random network.

Fig. 7 Sandstone pore network modularity

The sum of all subnets included in a network is the modular degree of this complex network. The calculation results are shown in Fig. 7. When the porosity is 17.1%, the pore network module of sandstone is approximately equal to 0.919, and the pore size of different sandstone pore networks is close to 1. It shows that sandstone pore network has good community structure. The nodes are grouped according to different societies, and the results are shown in Fig. 8, where the same community nodes are of the same color and size.

a. 14.1%                                      b. 16.9%

c. 17.1%                                         d. 24.6%

Fig. 8 Association structure of sandstone pore network

The numbers of nodes in each sandstone network are shown in Fig.9. When the porosity is 14.1% and 24.6%, the number of nodes in the pore network is less, and the number of nodes in each community is less than 200, which is about 1/10 of the total number of nodes. When the porosity is 16.9% and 17.1%, with the increase of network size, the number of nodes in a single community increases obviously. In the four samples, when the porosity is 16.9% and Modularity Class is 110, the maximum number of nodes in the network community is about 375, less than 1/20 of the total number of nodes. It shows that sandstone pore network distribution is uniform, and there is no phenomenon that some communities play a leading role in the whole sandstone network.

a. 14.1%                                                    b. 16.9%

c. 17.1%                                                    d. 24.6%

Fig. 9 Distribution of node numbers in network community of sandstone

- Conclusions

The topological structure of sandstone percolation network is the result of its physical and chemical properties, and the external stress environment. The results shows that the degree distribution of the nodes of the sandstone seepage network satisfies the single-peaked curve. When the porosity is 24.6%, the clustering coefficient is the largest, and the connectivity of the network is the best. In addition we sorted the nodes by the importance of the sandstone percolation network, and we find the sandstone pore network has large modularity and good uniformity. Although we do not know the quantitative relationship between the average path length and the absolute permeability of the key nodes to the porous sandstone, the conclusions of this paper may provide a angle for the exploitation of enhancing the oil and gas recovery.

Acknowledgement

Thanks for the help about the data and discussion from Prof. Martin Blunt in Imperial College London. This work is supported by the Priority Academic Program Development of Jiangsu Higher education Institutions and by the National Natural Science Foundation of China (Grant No. 11202228).

References

- M.J. Blunt, Flow in porous media — pore-network models and multiphase flow. Current Opinion in Colloid & Interface Science 6, 197-207, 2001.

- P.E. Øren and S. Bakke, Reconstruction of Berea sandstone and pore-scale modelling of wettability effects. Journal of Petroleum Science & Engineering 39, 177-199, 2003.

- M.J. Blunt, B. Bijeljic, H. Dong, et al, Pore-scale imaging and modelling. Adv.Water Resour 51, 197–216, 2013.

- Y. Ju, Y. Yang, Z. Song, W. Xu, A statistical model for porous structure of rocks. Sci. China Ser. E 51, 2040–2058, 2008.

- Øren P E and Bakke S. Process based reconstruction of sandstones and prediction of transport properties. Transport in Porous Media 46, 311-343, 2002.

- Xiucai Zhao. The study of reconstruction method of digital core and pore network model. China Petroleum University, (East China), 2009.

- A.Q. Raeini, B. Bijeljic, M.J. Blunt, Generalized network modeling: Network extraction as a coarse-scale discretization of the void space of porous media. Physical Review E 96, 013312, 2017.

- K. Shaina, E. Hesham, T. Carlos, et al, Assessing the utility of FIB-SEM images for shale digital rock physics. Advances in Water Resources 18, 1-14, 2015.

- A. Hajizadeh, A. Safekordi, F.A. Farhadpour, A multiple-point statistics algorithm for 3D pore space reconstruction from 2D images. Advances in Water Resources 34, 1256-1267, 2011.

- F. Civan, C.S. Rai, C.H.Sondergeld, Shale-gas permeability and diffusivity inferred by improved formulation of relevant retention and transport mechanisms. Transport in Porous Media 86, 925-944, 2011.

- T.Y. Zhang, D.L. Li, J.Q. Yang, et al, A study of the effect of pore characteristic on permeability with a pore network model. Petroleum Science & Technology 31, 1790-1796, 2013.

- Y. Ju, J. Zheng, E. Marcelo, et al, 3D numerical reconstruction of well-connected porous structure of rock using fractal algorithms. Computer methods in applied mechanics and engineering 279, 2014, 212-226.

- P. Wang, A.L. Barabasi, Understanding the Spreading Patterns of Mobile Phone Viruses. Science 324, 1071-1076, 2009.

- H. Jeong, B. Tombor, R. Albert, et al, The large-scale organization of metabolic networks. Nature 407, 651-654, 2000.

- E. Almaas, B. Kovacs, T. Vicsek, Z. N. Oltvai et al, Global organization of metabolic fluxes in the bacterium Escherichia coli. Nature 427, 839-843, 2004.

- F. Zhang, C. Hui, J. S. Terblanche, An interaction switch predicts the nested architecture of mutualistic networks. Ecol. Lett. 14, 797–803, 2011.

- P. Iassonov, T. Gebrenegus, M. Tuller, Segmentation of X-ray computed tomography images of porous materials: a crucial step for characterization and quantitative analysis of pore structures. Water Resour. Res. 45, W09415, 2009.

- W.L. Zhu, G.A. Gaetani, F. Fusseis, et al, Microtomography of Partially Molten Rocks: Three-Dimensional Melt Distribution in Mantle Peridotite, Science 332, 88-91, 2011.